Vortex Shedding Analysis Report

Generated: 2026-04-14 08:16:29

Input Parameters

| Parameter | Value |
| --- | --- |
| Material | A36 Steel |
| Young's modulus E (GPa) | 200.0 |
| Density (kg/m^3) | 7850 |
| Outer diameter D (mm) | 200.0 |
| Wall thickness t (mm) | 6.00 |
| Length L (m) | 6.00 |
| Damping ratio (zeta) | 0.0100 |
| Axial force (kN) | 0.0 |
| Wind climate | Port of Santos, Brazil |
| Mean wind (m/s) | 5.3 |

Calculated Section Properties

| Property | Value |
| --- | --- |
| Moment of inertia I (mm^4) | 17219936 |
| Cross-sectional area A (mm^2) | 3656.8 |
| Mass per unit length mu (kg/m) | 28.71 |
| Radius of gyration r (mm) | 68.6 |

Detailed Results — All Boundary Conditions

| Boundary condition | Mode | f (Hz) | V_crit (m/s) | F (kN) | sigma_max (MPa) |
| --- | --- | --- | --- | --- | --- |
| Fixed-Free (Cantilever) | 1 | 5.39 | 5.39 | 5.899 | 25.7 |
| Fixed-Free (Cantilever) | 2 | 33.69 | 33.69 | 230.423 | 348.4 |
| Hinged-Hinged (Simple) | 1 | 15.11 | 15.11 | 46.378 | 202.0 |
| Hinged-Hinged (Simple) | 2 | 60.49 | 60.49 | 742.805 | 1123.3 |
| Fixed-Fixed (Built-in) | 1 | 34.30 | 34.30 | 238.878 | 1040.4 |
| Fixed-Fixed (Built-in) | 2 | 94.48 | 94.48 | 1812.388 | 2740.7 |
| Free-Free | 1 | 34.30 | 34.30 | 238.878 | 1040.4 |
| Free-Free | 2 | 94.48 | 94.48 | 1812.388 | 2740.7 |
| Fixed-Hinged | 1 | 23.58 | 23.58 | 112.907 | 491.8 |
| Fixed-Hinged | 2 | 76.57 | 76.57 | 1190.203 | 1799.8 |
| Hinged-Free | 1 | 23.58 | 23.58 | 112.907 | 491.8 |
| Hinged-Free | 2 | 76.57 | 76.57 | 1190.203 | 1799.8 |

Damping Uncertainty (zeta 0.005 - 0.020)

| Boundary condition | Mode | sigma_min (MPa) | sigma_nom (MPa) | sigma_max (MPa) |
| --- | --- | --- | --- | --- |
| Fixed-Free (Cantilever) | 1 | 9.0 | 25.7 | 73.3 |
| Fixed-Free (Cantilever) | 2 | 122.7 | 348.4 | 994.2 |
| Hinged-Hinged (Simple) | 1 | 71.1 | 202.0 | 576.4 |
| Hinged-Hinged (Simple) | 2 | 395.4 | 1123.3 | 3205.0 |
| Fixed-Fixed (Built-in) | 1 | 366.3 | 1040.4 | 2968.6 |
| Fixed-Fixed (Built-in) | 2 | 964.8 | 2740.7 | 7820.1 |
| Free-Free | 1 | 366.3 | 1040.4 | 2968.6 |
| Free-Free | 2 | 964.8 | 2740.7 | 7820.1 |
| Fixed-Hinged | 1 | 173.1 | 491.8 | 1403.1 |
| Fixed-Hinged | 2 | 633.6 | 1799.8 | 5135.5 |
| Hinged-Free | 1 | 173.1 | 491.8 | 1403.1 |
| Hinged-Free | 2 | 633.6 | 1799.8 | 5135.5 |

Risk Assessment per Boundary Condition

| Boundary condition | Level | Wind percentile | Stress / yield |
| --- | --- | --- | --- |
| Fixed-Free (Cantilever) | WARNING | 55.8% | 10.3% |
| Hinged-Hinged (Simple) | CRITICAL | 99.7% | 80.8% |
| Fixed-Fixed (Built-in) | UNREACHABLE | 100.0% | 416.2% |
| Free-Free | UNREACHABLE | 100.0% | 416.2% |
| Fixed-Hinged | UNREACHABLE | 100.0% | 196.7% |
| Hinged-Free | UNREACHABLE | 100.0% | 196.7% |

Engineering Recommendations (worst case: CRITICAL)

• IMMEDIATE ACTION: Increase member size or add substantial bracing

• Consider higher grade steel (e.g., A572-50 → A992)

• Perform detailed fatigue analysis before deployment

Mode Shape Plots

Wind Speed Distribution